PARTE 3

module parte3(SW,KEY,LEDG,LEDR);

input [1:0]SW;

input [0:0]KEY;

output [8:0]LEDR;

output [0:0]LEDG;

parameter[8:0] A = 9'b000000001,

parameter[8:0] B = 9'b000000010,

parameter[8:0] C = 9'b000000100,

parameter[8:0] D = 9'b000001000,

parameter[8:0] E = 9'b000010000,

parameter[8:0] F = 9'b000100000,

parameter[8:0] G = 9'b001000000,

parameter[8:0] H = 9'b010000000,

parameter[8:0] I = 9'b100000000;

reg [8:0] estado_atual, prox_estado;

always @(posedge KEY[0])

if (~SW[0])

estado_atual <= A;

else

estado_atual <= prox_estado;

always @(estado_atual,SW[1])

case(estado_atual)

A: if(SW[1])

prox_estado = F;

else

prox_estado = B;

B: if(SW[1])

prox_estado = F;

else

prox_estado = C;

C: if(SW[1])

prox_estado = F;

else

prox_estado = D;

D: if(SW[1])

prox_estado = F;

else

prox_estado = E;

E: if(SW[1])

prox_estado = F;

else

prox_estado = E;

F: if(SW[1])

prox_estado = G;

else

prox_estado = B;

G: if(SW[1])

prox_estado = H;

else

prox_estado = B;

H: if(SW[1])

prox_estado = I;

else

prox_estado = B;

I: if(SW[1])

prox_estado = I;

else

prox_estado = B;

endcase

assign LEDG[0] = (estado_atual == (E|I));

endmodule;

PARTE 4

module ps2lab1(

input  CLOCK_50,

input  [3:0] KEY;

input  [17:0] SW;

output  [6:0] LCD;

output  [8:0] LEDG;

output  [17:0] LEDR;

input	PS2_DAT,

input	PS2_CLK,

inout  [35:0]  GPIO_0, GPIO_1

assign  GPIO_0    =  36'hzzzzzzzzz;

assign  GPIO_1    =  36'hzzzzzzzzz;

wire RST;

assign RST = KEY[0];

assign LEDR[17:0] = SW[17:0];

assign LEDG = 0;

wire reset = 1'b0;

wire [7:0] scan_code;

reg [7:0] history[1:4];

wire read, scan_ready;

oneshot pulser(

.pulse_out(read),

.trigger_in(scan_ready),

.clk(CLOCK_50)

);

keyboard kbd(

.keyboard_clk(PS2_CLK),

.keyboard_data(PS2_DAT),

.clock50(CLOCK_50),

.reset(reset),

.read(read),

.scan_ready(scan_ready),

.scan_code(scan_code)

);

lcd dsp0(history[1][3:0],HEX0);

always @(posedge scan_ready)

begin

history[4] <= history[3];

history[3] <= history[2];

history[2] <= history[1];

history[1] <= scan_code;

end

endmodule;

module LCDmodule(clk, RxD, LCD_RS, LCD_RW, LCD_E, LCD_DataBus);

input clk, RxD;

output LCD_RS, LCD_RW, LCD_E;

output [7:0] LCD_DataBus;

wire RxD_data_ready;

wire [7:0] RxD_data;

async_receiver deserialer(.clk(clk), .RxD(RxD), .RxD_data_ready(RxD_data_ready), .RxD_data(RxD_data));

assign LCD_RS = RxD_data[7];

assign LCD_DataBus = {1'b0, RxD_data[6:0]};   // sends only 7 bits to the module, padded with a '0' in front to make 8 bits

assign LCD_RW = 0;

reg [2:0] count;

always @(posedge clk) if(RxD_data_ready | (count!=0)) count <= count + 1;

reg LCD_E;

always @(posedge clk) LCD_E <= (count!=0);

endmodule